VLANS_Router-on-a-stick_static routing_ssh_VLSM

(In a school network)

Adressing scheme and table

Local network address 192.168.1.0/24 (Subnetted with VLSM)

Internet network 5.0.0.0/24

| Appliance | Interface | Ipv address | Subnet Mask | Default-gateway |
| --- | --- | --- | --- | --- |
| Internet Router | s0/1/0 | 192.168.1.202 | 255.255.255.252 | N/A |
|  | G0/0/0 | 5.0.0.1 | 255.255.255.0 | N/A |
| R1 | S0/1/0 | 192.168.1.201 | 255.255.255.252 | N/A |
|  | G0/0/1.10 | 192.168.1.161 | 255.255.255.224 | N/A |
|  | G0/0/1.20 | 192.168.1.129 | 255.255.255.224 | N/A |
|  | G0/0/0 | 192.168.1.1 | 255.255.255.128 | N/A |
| S1 | VLAN 99 | 192.168.1.194 | 255.255.255.248 | 192.168.1.193 |
| S2 (CCTV) | VLAN 99 | 192.168.1.195 | 255.255.255.248 | 192.168.1.193 |
| S3 | VLAN 30 | 192.168.1.2 | 255.255.255.128 | 192.168.1.1 |
| Staff | NIC | 192.168.1.190 | 255.255.255.224 | 192.168.1.161 |
| CCTV PC | NIC | 192.168.1.158 | 255.255.255.224 | 192.168.1.129 |
| CCTV Camera 1 | NIC | 192.168.1.157 | 255.255.255.224 | 192.168.1.129 |
| CCTV Camera 2 | NIC | 192.168.1.156 | 255.255.255.224 | 192.168.1.129 |
| CCTV Camera 3 | NIC | 192.168.1.155 | 255.255.255.224 | 192.168.1.129 |
| IOT Server | NIC | 192.168.1.154 | 255.255.255.224 | 192.168.1.129 |
| Laboratory | NIC | 192.168.1.126 | 255.255.255.128 | 192.168.1.1 |
| Internet server | NIC | 5.0.0.10 | 255.255.255.0 | 5.0.0.1 |

Vlan Table In the local Network

| Vlan ID | Name | Hosts number | Prot assigned | Net adress |
| --- | --- | --- | --- | --- |
| Vlan 10 | Staff | 15 | S1 f0/10 | 192.168.1.160/27 |
| Vlan20 | CCTV | 25 | S2f0/2-5 | 192.168.1.128/27 |
| VLAN 30 | Laboratory | 100 | S3 f0/1, SVI | 192.168.1.0/25 |
| Vlan 99 | Management | 2 | SVI(S1, S2) | 192.168.1.192/29 |
| Vlan 100 | Native | N/A | N/A | N/A |

Services

| Appliance | Basic_config | Security_configs |
| --- | --- | --- |
| R1 | Hostname: R1 No dns lookup Static_routing | Privileged_secret: Routerpa55 Console pass: Routerconpa55 User: Routeradmin Userpass: Routeradminpa55 Pass_length: 5 caracteres Encryption Block for 3 min, 5 attempts, 2 min Domain: School.com Ssh Static routes: - 0.0.0.0 0.0.0.0 192.168.1.202 |
| S1, S2, S3 | Hostname: S1, S2, S3 No dns lookup | Privileged_secret: switchpa55 Console pass: switchconpa55 User: switchadmin Userpass: switchadminpa55 Encryption Domain: School.com Ssh |
| Internet_Router | Static_routes | 0.0.0.0 0.0.0.0 192.168.1.201 |

How to test:

- Ping the internet server from any PC

- Connect on the iot server by typing its address in a web server and use the username Yannick and the password Dunia to Log into the server

- Explore both physical and logical topology to see how the network behaves and is structured

- SSH to any of the switches or to the router 1